# 722H MOTOR GRADER SPECIFICATIONS

- Equipped withShangChai SC11CB220G2B1 engine

- Sing-handle electro-hydraulic controlled power-shift T/M, 6forward &3reverse shifts makes operation easy and convenient

- Hydraulic lock, auto lock/unlock NO-SPIN differential ensures stability and powerful traction

- Ideal axel load allocation provides excellent stability while cutting hard surface

- Serviceable, efficient hydraulic system enables every part to be fully used

- Box-typed frame and advanced T/M enable it to complete heavy-duty work

- A wide working range is accomplished through flexible blade suspension system and articulated frame

| - Overall Dimension整机尺寸 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Length总长 |  |  |  |  |  |  |  |  |  |  |  |  |  | 9250mm |
| 2 | Width总宽 |  |  |  |  |  |  |  |  |  |  |  |  |  | 2600mm |
| 3 | Height(To the top of the cab)总高（到驾驶室顶部） |  |  |  |  |  |  |  |  |  |  |  |  |  | 3420mm |
| 4 | Wheel base轴距 |  |  |  |  |  |  |  |  |  |  |  |  |  | 6100mm |
| 5 | Tread轮距 |  |  |  |  |  |  |  |  |  |  |  |  |  | 2120(mm) |
| 6 | Min. turning radius 最小转弯半径 |  |  |  |  |  |  |  |  |  |  |  |  |  | 7.4（m） |
| Main Technical Specification主要技术参数 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Operating weight操作重量 |  |  |  |  |  |  |  |  |  |  |  |  | 16100kg |  |
| 2 | Blade length 刀板长度 |  |  |  |  |  |  |  |  |  |  |  |  | 4320mm |  |
| 3 | Blade height刀板高度 |  |  |  |  |  |  |  |  |  |  |  |  | 580mm |  |
| 4 | Shovel Blade thickness 铲刃板厚度 |  |  |  |  |  |  |  |  |  |  |  |  | 16mm |  |
| 5 | Max. lift above ground 最大提升高度 |  |  |  |  |  |  |  |  |  |  |  |  | 450mm |  |
| 6 | Max. grading depth 最大切削深度 |  |  |  |  |  |  |  |  |  |  |  |  | 535mm |  |
| 7 | Max. shoulder reach outside rear tire 铲刀最大侧伸距离 |  |  |  |  |  |  |  |  |  |  |  |  | 2400mm |  |
| 8 | Traveling speed(Km/h)，6 forward and 3 reverse行驶速度，前六后三 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | (1)forward前进 |  |  |  |  |  |  |  |  | 6.86/11.78/15.51/26.17/32.59/50.86 |  |  |  |  |  |
|  | (2) reverse后退 |  |  |  |  |  |  |  |  | 6.86/15.51/32.59 |  |  |  |  |  |
| Diesel engine发动机 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Model型号 |  |  |  |  | ShangChaiSC11CB220G2B1 engine |  |  |  |  |  |  |  |  |  |
| 2 | Type型式 |  |  |  |  | Direct injection. Turbocharged. Water cooling 直喷、增压、水冷 |  |  |  |  |  |  |  |  |  |
| 3 | Rated output额定功率 |  |  |  |  |  | 162 kW |  |  |  |  |  |  |  |  |
| 4 | Rated speed额定转速 |  |  |  |  |  | 2200(r/min) |  |  |  |  |  |  |  |  |
| 5 | Max. Torque最大扭矩 |  |  |  |  |  | 844(N.m)/(1400r/min) |  |  |  |  |  |  |  |  |
| Transmission case变速箱 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Model型号 |  |  |  | 6WG200（ZF） |  |  |  |  |  |  |  |  |  |  |
| 2 | Type形式 |  |  |  | power shift, single-shift lever, netural-shift lock device 动力换挡、单手柄、中位自锁 |  |  |  |  |  |  |  |  |  |  |
| 3 | Gear shift position档位 |  |  |  | 6 Forward and 3 reverse gears 前六后三 |  |  |  |  |  |  |  |  |  |  |
| Axle and Tire桥和轮胎 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  | Max. drawing force 最大牵引力 |  |  |  |  | 82.8KN（f=0.75） |  |  |  |  |  |  |  |
| 2 |  |  | Size of tire轮胎型号 |  |  |  |  | 17.5-25-14PR(standard) 标配 14.00-24-12PR(optional) 选配 |  |  |  |  |  |  |  |
| Working Device Hydraulic System工作液压系统 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | Model of oil pump(include steering pump) 油泵型号(包含转向泵) |  |  |  |  |  |  |  |  |  | CBGj1045/1025 |  |  |  |
| 2 |  | System pressure系统压力 |  |  |  |  |  |  |  |  |  | 17.5MPa |  |  |  |
| 3 |  | Model of distribution valve多路阀型号 |  |  |  |  |  |  |  |  |  | HUSCO 5000 |  |  |  |
| 4 |  | Model of oil motor回转马达型号 |  |  |  |  |  |  |  |  |  | EATON, 604-0663 |  |  |  |
| Steering System转向系统 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Type型号 |  |  | Fore Tiredeflexion前轮偏摆式转向 |  |  |  |  |  |  |  |  |  |  |  |
| 2 | Model of steering gear转向器型号 |  |  |  |  |  |  |  |  |  | BZZ1-E160B+FKA-163022 |  |  |  |  |
| 3 | System pressure系统压力 |  |  |  |  |  |  |  |  |  | 16MPa |  |  |  |  |
| Brake System制动系统 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Type of traveling brake 行车制动系统型式 |  |  |  |  |  |  |  | Hydraulic and inner-expanding brake 全液压内涨蹄式制动 |  |  |  |  |  |  |
| 2 | Model of brake pump制动泵型号 |  |  |  |  |  |  |  | CBGj1016-XF |  |  |  |  |  |  |
| 3 | System pressure系统压力 |  |  |  |  |  |  |  | 10MPa |  |  |  |  |  |  |
| 4 | Type of parking brake停车制动型式 |  |  |  |  |  |  |  | Mechanical and inner-expanding brake 机械式内涨蹄式制动器 |  |  |  |  |  |  |
| Oil Capacity用油量 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Fuel(diesel)燃油箱 |  |  |  |  |  |  |  |  |  |  |  |  | 370L |  |  |
| Engine lubricating oil发动机机油 |  |  |  |  |  |  |  |  |  |  |  |  | 27L |  |  |
| Oil for converter and gear box变速箱 |  |  |  |  |  |  |  |  |  |  |  |  | 28L |  |  |
| Oil for hydraulic system液压系统 |  |  |  |  |  |  |  |  |  |  |  |  | 65L |  |  |
| Oil for driving rear axles后桥 |  |  |  |  |  |  |  |  |  |  |  |  | 18L |  |  |
| Tandem box 平衡箱 |  |  |  |  |  |  |  |  |  |  |  |  | 90L |  |  |

3、722H平地机标准配置表

Standard configuration table for Motor Grader 722H

| 序号 NO. | 名称 Item | 型号 Model | 厂家 Manufacturer | 产地 Origin |
| --- | --- | --- | --- | --- |
| 1 | 发动机 Engine | SC11CB220G2B1 | 上柴 ShangChai | 中国 China |
| 2 | 变速箱 Transmission | 6WG200 | ZF | 中国 China |
| 3 | 驱动桥 Axle | 220H.8 | 常林 Changlin | 中国 China |
| 4 | 轮胎 Tire | 17.5-25-14PR | 三角轮胎 Triangle | 中国 China |
| 5 | 工作液压油泵: Working Pump | CBGj1045/1025 | 济南 JiNan | 中国 China |
| 6 | 转向装置 Steering System | BZZ1-E160B/ FKA-163022 | 镇江液压件厂 ZhenJiang | 中国 China |
| 7 | 回转马达 Rotary motor | 604-0663 | 伊顿 EATON | 美国 USA |
| 8 | 差速器: Differential | NO spin | 伊顿 EATON | 美国 USA |
| 9 | 多路阀 Valve | 5000 | HUSCO | 美国 USA |
| 10 | 液压锁（提升） Hydraulic lock(lift) | WBCDELU-03-B | LUEN | 意大利 Italy |
| 11 | 空调（选配） Air conditioner |  | 豫新 YUXIN | 中国 China |